# GUIDE SPECIFICATION

# Aluminum Coping Product

## SECTION 07620 - ROOF EDGE AND VENTILATION SYSTEMS

# Petersen Aluminum Corporation

# 1005 Tonne Road

# Elk Grove Village, IL 60007

# Telephone: (800) 323-1960 or

# (847-228-7150

# FAX: (847) 956-7960

** NOTE TO SPECIFIER ** Petersen Aluminum Corp; Roof edge and ventilation systems products.

This section is based on the products of Petersen Aluminum Corp, which is located at:
1005 Tonne Road

Elk Grove Village, IL 60007
Toll Free Tel: 800-722-7440
Fax: 800-722-7150
Email: info@pac-clad.com

Website: www.pac-clad.com

PART 1 – GENERAL

SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

Metal Fascia.

Metal Coping.

Metal Gravel Stop,

Metal Drip Edge.

Metal Coping Cap.

Vented and Non-Vented Eave and Fascia.

Ridge Vent.

Ballast paver terminations.

Accessories.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 06150 - Wood Decking.

Section 06160 - Sheathing: Plywood and oriented strand board.

Section 06200 - Wood Trim.

Section 07220 - Roof and Deck Insulation: Roof board insulation.

Section 07310 - Roof Shingles.

Section 07320 - Roof Tiles.

Section 07410 - Metal Roof and Wall Panels: Preformed metal roofing.

Section 07500 - Membrane Roofing.

    - Section 07620 - Sheet Metal Flashing and Trim.

    - Section 07720 - Roof Accessories: Soffit vents, roof pavers, etc.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

ANSI/SPRI ES-1 - Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

Florida Building Code, including the High Velocity Hurricane Zone.

Miami-Dade County NOA No. 18-0711.16.

Factory Mutual Research Corporation Approval Guide.

SPRI Single Ply Roofing Industry Standards.

  - SUBMITTALS

Submit under provisions of Section 01300.

Product Data: Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Show profiles, joining method, location of accessory items, anchorage and flashing details, adjacent construction interface, and dimensions.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

Selection Samples: For each finish product specified, two complete sets of color charts representing manufacturer's full range of available colors and patterns.

Verification Samples: For each finish product specified, two sample chips representing actual product, color, and patterns.

Manufacturer's Certificates: Certify products meet or exceed specified requirements.

  - QUALITY ASSURANCE

Manufacturer Qualifications: Company specializing in manufacturing Products specified in this section with minimum twenty five years documented experience.

Installer Qualifications: Company specializing in the installation of products specified in this section with minimum five years documented experience.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

Finish areas designated by Architect.

Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

Refinish mock-up area as required to produce acceptable work.

DELIVERY, STORAGE, AND HANDLING

Store products in manufacturer's unopened packaging until ready for installation.

Store materials in a dry, protected, well-vented area.

Remove protective plastic surface film immediately before installation.

Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - SEQUENCING

    - Ensure that information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

    - Coordinate installation with roof membrane manufacturer's installation instructions.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

WARRANTY

    - Provide the manufacturer's warranty specified under products for the roof edge system, when installed per manufacturer's instructions. Warranty will not exceed the life of the roof membrane on which the product was originally installed.

    - Provide a 30 year warranty for manufacturer approved PAC-CLAD 70 percent Kynar colors for the painted finish covering color fade, chalk, and film integrity. No finish warranty for anodized finishes.

PART 2 – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Petersen Aluminum Corp., which is located at: 1005 Tonne Road, Elk Grove Village, IL 60007; Toll Free Tel: 800-5722-7440; Fax: 800-722-7150; Email:pac-clad request info (info@pac-clad.com); Web:www.pac-clad.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

** NOTE TO SPECIFIER ** Edit the following paragraphs as required for the project requirements. Delete the paragraphs for the products not required.

  - COPING PRODUCTS

    - PAC-TITE Coping

      - Construction:

        - Metal:

- ** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 and 22 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvanized steel.

22 gauge galvalume steel.

        - Finish:

- ** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

          - Natural aluminum mill finish.

          - Clear anodized aluminum.

          - Black anodized Aluminum.

          - Dark Bronze anodized aluminum.

          - Champagne anodized aluminum.

          - Standard color Kynar-500 PAC-CLAD (aluminum and steel) as selected by the Architect from roof edge manufacturer's color chart.

          - Custom Kynar-500 color (aluminum and steel) as selected by the Architect.

        - Construction:

          - Coping cap: length of 12 feet 0 inches (3.65 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

          - Coping vertical face and back leg: 2-1/4 inch to 12-1/2 inch (57 mm to 3175 mm) manufactured to job requirements.

          - Concealed splice plates: 8 inch (203 mm) wide. Finish to match finish of coping cap with factory applied dual non-curing sealant strips.

          - Anchor/Support Cleat: 20 gauge pre-punched galvanized cleat with stainless steel spring mechanically locked to cleat normally 12 inch (305 mm) wide at 4 foot 0 inch (1.22 m) on center. Mechanically fastened as indicated and detailed.

        - Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

- ** NOTE TO SPECIFIER ** Select the Version(s) required from the following paragraphs and delete those not required.

      - Flat Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

          - Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code". – Min of .040 for 14” wall max, and min 050 for max 16” wall

      - Tapered Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code

          - Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code", Min of .040 for 14” wall max, and min 050 for max 16” wall

      - Over 6 inch to 12 inch Face Height, Tapered Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

      - Over 6 inch to 12 inch Face Height, Masonry Attached Tapered Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code

      - Larger than 16 inch Wall Size Tapered Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code

      - Existing Slope with Nailer Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code

      - Existing Slope without Nailer Version

        - Performance:

          - 20 Year, 120 mph Wind Warranty

          - Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code

Existing Slope without Nailer, Over 6 inch to 12 inch Face Height, Masonry Attached Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Double Tapered Coping:

Performance:

20 Year, 120 mph Wind Warranty.

    - PAC Continuous Cleat Coping

Construction:

Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvanized steel.

22 gauge galvalume steel

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum and steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum and steel) color as selected by the Architect.

Construction:

Coping cap: length of 12 feet 0 inches (3.65 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

Coping vertical face and back leg: 2-1/4 inch to 12-1/2 inch (57 mm to 3175 mm) manufactured to job requirements.

Concealed splice plates: 8 inch (203 mm) wide. Finish to match finish of coping cap with factory applied dual non-curing sealant strips.

Continuous Cleat: 20 gauge pre-punched galvanized cleat with stainless steel spring mechanically locked to cleat normally 12 inch (305 mm) wide at 4 foot 0 inch (1.22 m) on center. Mechanically fastened as indicated and detailed.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

** NOTE TO SPECIFIER ** Select the Version(s) required from the following paragraphs and delete those not required.

Flat Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Tapered Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 1 Standard to comply with the International Building Code.

Over 6 inch to 12 inch Face Height, Tapered Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Over 6 inch to 12 inch Face Height, Masonry Attached Tapered Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Larger than 16 inch Wall Size Tapered Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Existing Slope with Nailer Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Existing Slope without Nailer Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Existing Slope without Nailer, Over 6 inch to 12 inch Face Height, Masonry Attached Version

Performance:

20 Year, 120 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Double Tapered Coping:

Performance:

20 Year, 120 mph Wind Warranty.

    - PAC-TITE Gold Coping

Construction:

Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 and 22 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvanized steel.

22 gauge galvalume steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum and steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum and steel) color as selected by the Architect.

Construction:

Coping cap: Length of 12 feet 0 inches (3.65 m), widths to 30 inch (762 mm) manufactured to job requirements. True radii may be built to template.

Coping vertical face and back leg: 2-3/4 inch to 12 inch (69.85 mm to 304.8 mm) manufactured to job requirements.

Concealed splice plates: 8 inch (203 mm) wide. Finish to match finish of coping cap with factory applied dual non-curing sealant strips.

Anchor/Support Cleat: 16 gauge pre-punched galvanized cleat with stainless steel spring mechanically locked to cleat normally 12 inch (305 mm) wide at 3 foot 0 inch (.9144 m) on center. Mechanically fastened as indicated and detailed.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

** NOTE TO SPECIFIER ** Select the Version(s) required from the following paragraphs and delete those not required.

Tapered Version

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code", Min of .050, 6” max outside face, 4” max inside face; 25 1/8” max width

Over 6 inch to 12 inch Face Height, Tapered Version

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

Larger than 16 inch Wall Size Tapered Version

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

    - Welded Radius Coping

Construction:

Metal:

Inside and Outside Faces: .063 inch (1.6 mm) aluminum.

Top of coping: 080 inch (2.03 mm).

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum and steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum and steel) color as selected by the Architect.

Construction:

Coping cap: length of 4 feet 0 inches (1.22 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

Coping vertical face and back leg: 2-1/4 inch to 12 (57 mm to 305 mm) manufactured to job requirements.

Concealed splice plates: 8 inch (203 mm) wide. Finish to match finish of coping cap with factory applied dual non-curing sealant strips.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

Anchor/Support Cleat: 20 gauge pre-punched galvanized cleat with stainless steel spring mechanically locked to cleat normally 12 inch (305 mm) wide at 4 foot 0 inch (1.22 m) on center. Mechanically fastened as indicated and detailed.

    - Quicklock Radius Coping

Construction:

Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 and 22 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvalume steel.

22 gauge galvalume steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum and steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum and steel) color as selected by the Architect.

Construction:

Coping cap: length of 4 feet 0 inches (1.22 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

Coping vertical face and back leg: 2-1/4 inch to 12 (57 mm to 305 mm) manufactured to job requirements.

Concealed splice plates: 8 inch (203 mm) wide. Finish to match finish of coping cap with factory applied dual non-curing sealant strips.

Anchor/Support Cleat: 20 gauge pre-punched galvanized cleat with stainless steel spring mechanically locked to cleat normally 12 inch (305 mm) wide at 4 foot 0 inch (1.22 m) on center. Mechanically fastened as indicated and detailed.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

    - Arched Coping: Full, snap-on design with no crimping. Pre-punched fastening holes ensure proper attachment to the building substrate.

Metal:

.100 inch (2.54 mm) mill finished aluminum.

Aluminum Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish, painted to PAC-CLAD Kynar 500 colors or custom Kynar color.

Construction:

Coping cap: length of 4 feet 0 inches (1.22 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

Splice Plates: Interlocking, concealed splice plate design prevents splice plate migration under the fascia panels.

Corrosion resistant, stainless steel springs are factory attached to the anchor clips and provide long-term, positive spring loading. No adhesives are required.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

Performance:

5 Year Workmanship Warranty.

    - Arched Coping: Top Fasten Version: Pre-punched fastening holes ensure proper attachment to the building substrate.

Metal:

.100 inch (2.54 mm) mill finished aluminum.

Aluminum Finish:

Natural aluminum mill finish,painted to PAC-CLAD Kynar 500 colors or custom Kynar color

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Construction:

Coping cap: length of 4 feet 0 inches (1.22 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

Splice Plates: Interlocking, concealed splice plate design prevents splice plate migration under the fascia panels.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

Performance:

5 Year Workmanship Warranty.

    - Arched Coping: Snap-On Version with 24 gauge (.65 mm) spring clips 4 foot (1.22 m) o.c.

Metal:

.100 inch (2.54 mm) mill finished aluminum.

Aluminum Finish:

Natural aluminum mill finish,painted to PAC-CLAD Kynar 500 colors or custom Kynar color

Natural aluminum mill finish,painted to PAC-CLAD Kynar 500 colors or custom Kynar color ** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Construction:

Coping cap: length of 4 feet 0 inches (1.22 m), widths to 24 inch (101.6 mm) manufactured to job requirements. True radii may be built to template.

Splice Plates: Interlocking, concealed splice plate design prevents splice plate migration under the fascia panels.

Fasteners: 1-1/2 inch (38 mm) Stainless Steel with driver.

Performance:

5 Year Workmanship Warranty.

Factory Mutual 1-90 approved for wind up lift protection.

  - ACCESSORIES

Miters and end caps shall be fabricated by manufacturer to suit the conditions indicated on the Drawings.

Provide fasteners consistent with manufacturer's instructions for each product that is suitable for the substrate to which it is being installed.

- – EXECUTION

  - EXAMINATION

Do not begin installation until substrates have been properly prepared.

Verify that the substrate is dry, clean and free of foreign matter.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

PREPARATION

Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

Verify the manufacturer's roof edge details for accuracy to fit the assembly prior to fabrication.

INSTALLATION

Install in accordance with manufacturer's installation instructions.

Use provided fasteners consistent with manufacturer's instructions, suitable for the substrate to which it is being installed.

Install water cut-off, as recommended by the membrane manufacturer, under the anchor bar.

PROTECTION

Protect installed products until completion of project.

Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION