# GUIDE SPECIFICATION

# Aluminum Fascia Product

## SECTION 07620 - ROOF EDGE AND VENTILATION SYSTEMS

# Petersen Aluminum Corporation

# 1005 Tonne Road

# Elk Grove Village, IL 60007

# Telephone: (800) 323-1960 or

# (847-228-7150

# FAX: (847) 956-7960

** NOTE TO SPECIFIER ** Petersen Aluminum Corp; Roof edge and ventilation systems products.
This section is based on the products of Petersen Aluminum Corp, which is located at:
1005 Tonne Road

Elk Grove Village, IL 60007
Toll Free Tel: 800-722-7440
Fax: 800-722-7150
Email: info@pac-clad.com

Website: www.pac-clad.com

- GENERAL

SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

Metal Fascia.

Metal Coping.

Metal Gravel Stop,

Metal Drip Edge.

Metal Coping Cap.

Vented and Non-Vented Eave and Fascia.

Ridge Vent.

Ballast paver terminations.

Accessories.

RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

Section 06150 - Wood Decking.

Section 06160 - Sheathing: Plywood and oriented strand board.

Section 06200 - Wood Trim.

Section 07220 - Roof and Deck Insulation: Roof board insulation.

Section 07310 - Roof Shingles.

Section 07320 - Roof Tiles.

Section 07410 - Metal Roof and Wall Panels: Preformed metal roofing.

Section 07500 - Membrane Roofing.

Section 07620 - Sheet Metal Flashing and Trim.

Section 07720 - Roof Accessories: Soffit vents, roof pavers, etc.

REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

ANSI/SPRI/FM 4435 ES-1 - Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

Florida Building Code, including the High Velocity Hurricane Zone.

Miami-Dade County NOA No. 18-0711.16.

Factory Mutual Research Corporation Approval Guide.

SPRI Single Ply Roofing Industry Standards.

SUBMITTALS

Submit under provisions of Section 01300.

Product Data: Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Show profiles, joining method, location of accessory items, anchorage and flashing details, adjacent construction interface, and dimensions.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

Selection Samples: For each finish product specified, two complete sets of color charts representing manufacturer's full range of available colors and patterns.

Verification Samples: For each finish product specified, two sample chips representing actual product, color, and patterns.

Manufacturer's Certificates: Certify products meet or exceed specified requirements.

QUALITY ASSURANCE

Manufacturer Qualifications: Company specializing in manufacturing Products specified in this section with minimum twenty five years documented experience.

Installer Qualifications: Company specializing in the installation of products specified in this section with minimum five years documented experience.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

Finish areas designated by Architect.

Do not proceed with remaining work until workmanship, color, and sheen are approved by Architect.

Refinish mock-up area as required to produce acceptable work.

DELIVERY, STORAGE, AND HANDLING

Store products in manufacturer's unopened packaging until ready for installation.

Store materials in a dry, protected, well-vented area.

Remove protective plastic surface film immediately before installation.

Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

SEQUENCING

Ensure that information required for installation of products of this section are furnished to affected trades in time to prevent interruption of construction progress.

Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

Coordinate installation with roof membrane manufacturer's installation instructions.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

WARRANTY

Provide the manufacturer's warranty specified under products for the roof edge system, when installed per manufacturer's instructions. Warranty will not exceed the life of the roof membrane on which the product was originally installed.

Provide a 30 year warranty for manufacturer approved prefinished PAC-CLAD 70 percent Kynar colors for the painted finish covering color fade, chalk, and film integrity. No finish warranty available on anodized material.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Petersen Aluminum Corp., which is located at: 1005 Tonne Road, Elk Grove Village, IL 60007; Toll Free Tel: 800-5722-7440; Fax: 800-722-7150; Email:pac-cladrequest info (info@pac-clad.com); Web:www.pac-clad.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01600.

** NOTE TO SPECIFIER ** Edit the following paragraphs as required for the project requirements. Delete the paragraphs for the products not required.

ROOF EDGE PRODUCTS

PAC-TITE Angular Fascia: Decorative metal fascia with continuous extruded aluminum bar.

Construction:

Fascia metal gauge:

** NOTE TO SPECIFIER ** Select one of the following metal paragraphs and delete the one not required. Note that 24 gauge galvanized steel is available unfinished only.

.040 inch (1 mm) thick formed aluminum.

24 gauge galvalume steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum and steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum or steel) color as selected by the Architect, minimum quantities apply

Fascia: Standard 12 feet 0 inches (3.65 m) lengths.

Extruded bar: Shall be continuous 6063-T6 alloy aluminum at 12 feet 0 inches (3.65 m) standard lengths with pre-punched slotted holes. All bar miters are welded.

Injection Molded EPDM Bar Splice to allow thermal movement expansion of extruded aluminum anchor bar.

Fasteners: 2 inch (51 mm) stainless steel with driver.

** NOTE TO SPECIFIER ** Select the Version(s) required the following paragraphs and delete those not required.

Fully Adhered or Mechanically Attached Single-Ply Version

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. AF-55 -5-1/2 inch (139.7 mm) face height).

AF-40.

AF-55.

AF-70.

AF-85.

Performance:

Lifetime, 215 mph Wind Warranty.

Tested per ANSI/SPRI ES-1 Standard to a design pressure of 275 lbs./ft2to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code".

Built-Up or Modified Version

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. AMF-55 -5-1/2 inch face height).

AFM-55.

AFM-70

AFM-85.

Performance:

Lifetime, 215 mph Wind Warranty.

Tested per ANSI/SPRI ES-1 Standard to a design pressure of 275 lbs./ft2to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code".

PAC-TITE WT Fascia: Two-part assembly consisting of a continuous extruded aluminum bar and exterior fascia with fasteners.

Construction:

Fascia Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required.

.050 inch (1.27 mm) thick-formed aluminum.

.063 inch (1.6 mm) thick-formed aluminum.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD aluminum as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 aluminum color as selected by the Architect, minimum quantities apply.

Fascia: standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates.

Extruded bar: Shall be continuous 6063-T6 alloy aluminum at 12 feet 0 inches (3.65 m) standard lengths with pre-punched slotted holes. All bar miters are welded.

Injection Molded EPDM Bar Splice to allow thermal movement expansion of extruded aluminum anchor bar.

Fasteners: 2 inch (51 mm) stainless steel with driver.

** NOTE TO SPECIFIER ** Select the Version(s) required the following paragraphs and delete those not required.

Fully Adhered or Mechanically Attached Single-Ply Version:

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. HG-AF--55 -5-1/2 inch (139.7 mm) face height).

WT-AF-40.

WT-AF-55

WT-AF-70.

Performance:

Lifetime, 215 mph Wind Warranty.

Tested per ANSI/SPRI ES-1 Standard to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

Built-Up or Modified Version:

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. AF-55 -5-1/2 inch (139.7 mm) face height).

WT-AFM-55.

WT-AFM-70.

Performance:

Lifetime, 215 mph Wind Warranty.

Tested per ANSI/SPRI ES-1 Standard to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

PAC-TITE WT Extended Fascia: A three-part assembly consisting of a continuous extruded aluminum bar, exterior fascia and continuous formed galvanized steel cleat with fasteners.

Construction:

Fascia Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required.

.050 inch (1.27 mm) thick-formed aluminum.

.063 inch (1.6 mm) thick-formed aluminum.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD aluminum as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 aluminum color as selected by the Architect, minimum quantities apply.

Fascia: standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates.

Extruded Bar: Shall be continuous 6063-T6 alloy aluminum in 12 feet 0 inches (3.65 m) standard lengths with pre-punched slotted holes. All bar miters are welded.

Injection Molded EPDM Bar Splice to allow thermal movement expansion of extruded aluminum anchor bar.

Fasteners: 2 inch (51 mm) stainless steel and 1/4 inch (6 mm) by 1-1/4 inch (32 mm) masonry/wood with driver.

Anchor Bar Cleat: Standard 12 feet 0 inches (3.65 m) lengths of commercial type 20 gauge G-90 galvanized steel with pre-punched holes.

** NOTE TO SPECIFIER ** Select the Version(s) required the following paragraphs and delete those not required.

Fully Adhered or Mechanically Attached Single-Ply Version:

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. AEF-85 -8-1/2 inch (216 mm) face height).

AEF-85.

AEF-100.

AEF-115.

AEF-130.

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI ES-1 Standard to a design pressure of 176 lbs./ft2to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code".

Built-Up or Modified Version

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. AEFM-85 - 8-1/2 inch (216 mm) face height).

AEFM-85.

AEFM-100.

AEFM-115.

AEFM-130.

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI ES-1 Standard to a design pressure of 176 lbs./ft2to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code".

PAC-TITE WT Canted Fascia: Three-part assembly consisting of a continuous extruded aluminum bar, exterior fascia and continuous formed galvanized steel cleat with fasteners.

Construction:

Fascia Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required.

.050 inch (1.27 mm) thick-formed aluminum.

.063 inch (1.6 mm) thick-formed aluminum.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD aluminum as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 color as selected by the Architect.

Fascia: standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates.

Extruded Bar: Shall be continuous 6063-T6 alloy aluminum in 12 feet 0 inches (3.65 m) standard lengths with pre-punched slotted holes. All bar miters are welded.

Injection Molded EPDM Bar Splice to allow thermal movement expansion of extruded aluminum anchor bar.

Fasteners: 2 inch (51 mm) stainless steel and 1/4 inch (6 mm) by 1-1/4 inch (32 mm) masonry/wood with driver.

Waterdam: Standard 12 feet 0 inches (3.65 m) lengths of commercial type 24 gauge G-90 galvanized steel with pre-punched slotted holes.

Anchor Bar Cleat: Standard 12 feet 0 inches (3.65m) lengths of commercial type 20 gauge G-90 galvanized steel with pre-punched holes.

** NOTE TO SPECIFIER ** Select the Version(s) required the following paragraphs and delete those not required.

Galvanized Waterdam Version

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. AEFC-1052 - 10-1/2 inch face height).

AEFC-552 (5 ½”)

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI ES-1 Standard to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

PAC-TITE Extended Canted Fascia:

Construction:

Fascia Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required.

.050 inch (1.01 mm) aluminum.

.063 inch (1.6 mm) aluminum.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD aluminum as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 color as selected by the Architect.

Extruded aluminum anchor bar.

Interlocking, concealed splice plate design prevents splice plate migration under the fascia panels.

Injection Molded EPDM Bar Splice to allow thermal movement expansion of extruded aluminum anchor bar.

Fasteners: 2 inch (51 mm) stainless steel and 1/4 inch (6 mm) by 1-1/4 inch (32 mm) masonry/wood with driver.

Provided with anchor bar EPDM splice plates and fasteners.

** NOTE TO SPECIFIER ** Select the Version(s) required the following paragraphs and delete those not required.

Galvanized Waterdam Version

Sizes:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required.

8-1/2 inch (215.9 mm).

10 inch (254 mm).

11-1/2 inch (292.1 mm).

13 inch (330.2 mm).

Performance:

Lifetime, 215 mph Wind Warranty

Tested per ANSI/SPRI ES-1 Standard to comply with the International Building Code.

Factory Mutual approved for wind up lift protection.

PAC-LOC 2000 Fascia: Decorative metal fascia with continuous galvanized steel waterdam to terminate single-ply roofing at perimeter.

Construction:

Fascia Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 and 22 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvalume steel.

22 gauge galvalume steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum or steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum or steel) color as selected by the Architect.

Fascia: standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates.

Waterdam: Continuous 24 gauge commercial type G-90 galvanized steel at 12 feet 0 inches (3.65 m) standard lengths.

Fasteners: 1-1/4 inch (32 mm) Galvanized Ring Shank Roofing Nails.

** NOTE TO SPECIFIER ** Select the Version(s) required the following paragraphs and delete those not required.

Single-Ply Application, Snap-On Version with 24 gauge (.65 mm) spring clips 4 foot (1.22 m) o.c.

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. FA-1052 - 10-1/2 inch face height).

FA-50. (5 ¼”)

FA-65. (6 ¾”)

FA-80. (8 ¼”)

FA-95. (9 ¾”)

FA-110. (11 ¼”)

FA-125. (12 ¾”)

Performance:

20 Year, 110 mph Wind Warranty.

Face heights of 5-1/4 inch to 8-1/4 inch are tested per ANSI/SPRI ES-1 Standard to comply with the International Building Code.

PAC-LOC 3000 Fascia: Decorative metal fascia with continuous galvanized steel waterdam to terminate built-up or modified roofing at perimeter.

Construction:

Fascia Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 and 22 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvalaume steel.

22 gauge galvalume steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum or steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum or steel) color as selected by the Architect, minimum quantities apply

Fascia: standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates.

Waterdam: Shall be continuous 24 gauge commercial type G-90 galvalume steel at 12 feet 0 inches (3.65 m) standard lengths.

Fasteners: 1-1/4 inch (32 mm) galvanized ring shank roofing nails.

Built-Up or Modified Application, Snap-On Version with 24 gauge (.65mm) spring clips 4 feet (1.22 m) o.c.

Model:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete those not required. Note that fascia model number denotes height in inches; (i.e. FA-1052 - 10-1/2 inch face height).

FA-350. (5 ½”)

FA-365. (6 ¾”)

FA-380. (8 ¼”)

FA-395. (9 ¾”)

FA-3110. (11 ¼”)

FA-3125. (12 ¾”)

Performance:

20 Year, 110 mph Wind Warranty.

Face heights of 5-1/4 inch to 8-1/4 inch are tested per ANSI/SPRI ES-1 Standard to to comply with the International Building Code.

PAC Snap Edge: Decorative metal fascia to terminate single ply roofing at perimeter.

Construction:

Cover Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 and 22 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvanized steel.

22 gauge galvanized steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum or steel) as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 color as selected by the Architect; minimum quantities apply

Fascia Covers: Standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates. Cover height shall be:

** NOTE TO SPECIFIER ** Select one of the following paragraphs and delete the ones not required.

4 inches.

5 inches.

6-1/2 inches.

8 inches.

As indicated on the Drawings.

Formed Rail: Shall be 20 gauge commercial type G-90 galvanized steel at 12 feet 0 inches (3.65 m) standard lengths with pre-punched slots 12 inches o.c. Provided with stainless steel spring mechanically locked to clips 4 foot 0 inch (1.22 m) on center.

Fasteners: 1-1/4 inch (32 mm) galvanized ring shank roofing nails.

Performance:

20 Year, 110 mph Wind Warranty.

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to a design pressure of 396 lbs./sf to comply with the International Building Code.

Florida Building Code Approved.

Factory Mutual approved with Aluminum Rail.

Miami-Dade Approved (No. 18-0711.16 12/11/23) to comply with the "High Velocity Hurricane Zone of the Florida Building Code".

PAC Edge Systems One Drip Edge: Features an "L" shaped continuous cleat with pre-slotted fastening holes.

Metal:

** NOTE TO SPECIFIER ** Select Metal type and thickness required from the following paragraphs and delete those not required. Note that 24 gauge galvanized steel is available unfinished only.

.040 inch (1.01 mm) aluminum.

.050 inch (1.27 mm) aluminum.

.063 inch (1.6 mm) aluminum.

24 gauge galvalume steel.

Finish:

** NOTE TO SPECIFIER ** Select one of the following finish paragraphs and delete the one not required.

Natural aluminum mill finish.

Clear anodized aluminum.

Black anodized Aluminum.

Dark Bronze anodized aluminum.

Extra Dark Bronze anodized aluminum.

Champagne anodized aluminum.

Standard color Kynar-500 PAC-CLAD (aluminum or steel)  as selected by the Architect from roof edge manufacturer's color chart.

Custom Kynar-500 (aluminum or steel) color as selected by the Architect; minimum quantities apply.

Fascia: standard 12 feet 0 inches (3.65 m) lengths with matching concealed joint splice plates.

Splice Plates and 22 gauge galvanized continuous cleats with slotted holes are included

Performance:

5 Year Workmanship Warranty.

Tested per ANSI/SPRI/FM 4435 ES-1 Standard to comply with the International Building Code.

ACCESSORlES

Miters and end caps shall be fabricated by manufacturer to suit the conditions indicated on the Drawings.

Provide fasteners consistent with manufacturer's instructions for each product that is suitable for the substrate to which it is being installed.

- EXECUTION

EXAMINATION

Do not begin installation until substrates have been properly prepared.

Verify that the substrate is dry, clean and free of foreign matter.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

PREPARATION

Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

Verify the manufacturer's roof edge details for accuracy to fit the assembly prior to fabrication.

INSTALLATION

Install in accordance with manufacturer's installation instructions.

Use provided fasteners consistent with manufacturer's instructions, suitable for the substrate to which it is being installed.

Install water cut-off, as recommended by the membrane manufacturer, under the anchor bar.

PROTECTION

Protect installed products until completion of project.

Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION