# GUIDE SPECIFICATION

# Flush Soffit

August 2024

## SECTION 07415- PREFORMED ALUMINUM SOFFIT PANELS, SOLID AND VENTED

– GENERAL

DESCRIPTION OF WORK

This section covers the pre-finished, prefabricated Factory Manufactured Aluminum or Steel Soffit System. All metal trim, accessories, fasteners, insulation and sealants indicated on the drawings are part of this section

Related Work Specified Elsewhere

Roof Deck structural steel, flat roof systems, preformed metal standing seam roofing, perimeter edge systems, roof hatches, firestopping not included in this section

QUALITY ASSURANCE

Petersen Aluminum Corp products establish a minimum of quality required. Specifier Note: Select from Factory Location below:

Petersen Aluminum Corp, Elk Grove Village, IL, 800-323-1960

Petersen Aluminum Corp, Annapolis Junction, MD, 800-344-1400

Petersen Aluminum Corp, Tyler, TX, 800-441-8661

Petersen Aluminum Corp, Acworth, GA, 800-272-4482

Manufacturers and erector shall demonstrate experience of a minimum of five (5) years in this type of project.

Sheet Metal Industry Standard: Comply with Sheet Metal and Air Conditioning Contractors National Association (SMACNA) Architectural Sheet Metal Manual.

Panels shall be factory-produced only. No portable, installer-owned or installer-rented machines will be permitted.

SUBSTITUTIONS

The material, products and equipment specified in this section establish a standard for required function, dimension, appearance and quality to be met by any proposed substitution.

SYSTEM DESCRIPTION

Material to comply with Specifier Note: Choose from the below based on required gauge:

ASTM B209 Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate - .032 & .040 Aluminum

ASTM A 653 Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy Coated by the Hot-Dip Process - 24 GA

ASTM A792/A792M Standard Specification for Sheet Steel, 55% Aluminum-Zinc Alloy Coated by the Hot-Dip process – 22 GA

SOFFIT SYSTEM PERFORMANCE TESTING

Soffit System shall be designed to meet Standard Building Code wind load requirements.

The Soffit System shall be designed to meet applicable Local Building Code and the Soffit System shall have been tested by the Manufacturer per ASTM E-330 and have the applicable Load Tables published from this Air Bag testing for negative loads.

WARRANTIES

Finish warranty: Manufacturer’s standard form in which manufacturer agrees to repair finish or replace soffit panels that show evidence of deterioration of factory-applied finish within specified warranty period.

Exposed Panels Finish – deterioration includes the following:

Color fading more than 5 hunter units when tested according to ASTM D 2244

Chalking in excess of a No. 8 rating when tested according to ASTM D 4214

Cracking, checking, peeling or failing paint to adhere to bare metal.

Warranty Period: 30 Years from the date of substantial completion

SUBMITTALS

Furnish detailed drawings showing profile and gauge of exterior sheets, location and type of fasteners, location, gauges, shape and method of attachment of all trim locations and types of sealants, and any other details as may be required for a weather-tight installation.

Provide finish samples of all specified colors.

LEED Submittals

Product data for Credit MR 4.1 and credit MR 4.2: Indicating the percentages by weight of postconsumer and preconsumer recycled content for products having recycled content

DELIVERY, STORAGE AND HANDLING

Deliver components, sheets, metal soffit panels and other manufactured items so as not to be damaged or deformed. Package metal soffit panels for protection during transportation and handling.

Unload, store and erect metal soffit panels in a manner to prevent bending, warping, twisting and surface damage.

Stack metal soffit panels on platforms or pallets, covered with suitable weather tight and ventilated covering. Store metal soffit panels to ensure dryness. Do not store metal soffit panels in contact with other materials that might cause staining, denting or other surface damage.

Protect strippable protective coating on any metal coated product from exposure to sunlight and high humidity, except to the extent necessary for material installation.

PROJECT CONDITIONS

Weather Limitations: proceed with installation only when existing and forecasted weather conditions permit metal roof panel work to be performed.

Field Measurements: Verify actual dimensions of construction contiguous with metal roof panels by field measurements before fabrication

– PRODUCTS

PANEL DESIGN

Soffit panels shall be (specifier to choose) Flush or Reveal Panel, in 7”, 11” or 12” widths with 1” height. Flush type panels can be specified in solid or with venting (.032 aluminum panels only) in the specified color (s).

Panels to be produced (specifier to choose) smooth, with one pencil rib, with two pencil ribs.  Specifier note: Factory standard is smooth unless specified.

Forming: Use continuous end rolling method. No end laps on panels. No portable roll forming machines will be permitted on this project, no installer-owned or installer-rented machines will be permitted. It is the intent of the Architect to provide Factory-Manufactured panel systems only for this project.

ACCEPTABLE MANUFACTURERS

This project is specified around the roofing product of Petersen Aluminum Corporation, Flush Soffit Panel.

Other acceptable manufacturers, if they comply with specification:

IMETCO, Tucker, GA, Soffit Panel only

ATAS Aluminum, Allentown, PA, “Wind-Lock” Panel only

MATERIALS AND FINISHES

Preformed metal panels shall be fabricated of (specifier to choose) 24 GA Galvalume steel, 22 GA galvalume steel, .032” thick 3105-H14 aluminum, .040” thick  3105-H14 aluminum.

Color shall be PAC-CLAD Kynar 500 stocked__________________________

Finishes: Finish shall be Kynar 500 or Hylar 5000 Fluorocarbon coating with a top side film thickness of 0.70 to 0.90 mil over 0.25 to 0.31 mil prime coat to provide a total dry film thickness of 0.95 to 1.25 mil. Finish shall conform to tests for adhesion, flexibility and longevity as specified by Kynar 500 or Hylar 5000 finish supplier.

Panels in 032 aluminum only can be specified as “wide” or “narrow” vented. Steel panels may not be vented.

Field protection must be provided by the Contractor at the job site, so material is not exposed to weather and moisture.

If any strippable film coating is applied to any pre-finished panels or materials for protection during shipping, strippable film shall be removed prior to installation.

Forming: use continuous and rolling method. No end laps on panels. No “portable roll forming” machines will be permitted on this project; no installer-owned or installer-rented machines shall be permitted. It is the intent of the Architect to provide Factory-Manufactured soffit systems only for this project.

Trim: Trim shall be fabricated of the same material and finish to match the profiled sheeting and press broken in lengths of 10 – 12 feet. Trim shall be formed only by the manufacturer or their approved dealer. Trim to be erected in overlapped condition. Use lap strips only as indicated on drawings. Miter conditions shall be factory welded material to match the sheeting.

Accessories/Fasteners: Fasteners shall be of type, material, size, corrosion resistance, holding power and other properties required to fasten miscellaneous framing members to substrates. Accessories and their fasteners shall be capable of resisting the specified design wind uplift forces and shall allow for thermal movement of the wall panel system. Exposed fasteners shall not restrict free movement of the roof panel system resulting from thermal forces, except at designed points of roof panel fixity

Miscellaneous Metal Sub framing and Furring: ASTM C 645, cold-formed, metallic-coated steel sheet, ASTM A 653/A 653M, G90 hot-dip galvanized coating designation or ASTM A 792/A 792M, Class AZ50 aluminum-zinc-alloy coating designation unless otherwise indicated. Provide manufacturer's standard sections as required for support and alignment of metal panel system.

Zees: Where required by design of primary structural framing system, zees shall be used to span between beams and/or other joists. Thermally responsive base and top clips shall be fastened to the zees on 12” centers.

Insulation: See Section 07210: Building Insulation.

SEALANTS

Provide two-part polysulfide class B non-sag type for vertical and horizontal joints or

One part polysulfide not containing pitch or phenolic extenders or

Exterior grade silicone sealant recommended by roofing manufacturer or

One-part non-sag, gun grade exterior type polyurethane recommended by the roofing manufacturer.

FABRICATION

Comply with dimensions, profile limitations, gauges and fabrication details shown and if not shown, provide manufacturer’s standard product fabrication.

Fabricate components of the system in factory, ready for field assembly.

Fabricate components and assemble units to comply with fire performance requirements specified.

Apply specified finishes in conformance with manufacturer’s standard, and according to manufacturer’s instructions.

– EXECUTION

INSPECTION

Examine alignment of structural steel and related supports, primary and secondary roof framing, solid roof sheathing, prior to installation.

For the record, prepare a written report, endorsed by the installer, listing conditions detrimental to performance of the Work.

Proceed with installation only after unsatisfactory conditions have been corrected.

FASTENERS

Secure units to support

Place fasteners as indicated in manufacturer’s standards.

INSTALLATION

Panels shall be installed plumb and true in a proper alignment and in relation to the structural framing. The erector must have at least five years successful experience with similar applications.

Install soffit panels, fasteners, trim and related sealants in accordance with approved shop drawings and as may be required for a weather-tight, complete and architecturally pleasing installation.

Remove all strippable coating and provide a dry wipe down cleaning of the panels as they are erected.

Panels attached to any TREATED LUMBER MUST HAVE AN APPROPRIATE VAPOR BARRIER INSTALLED OVER THE TREATED LUMBER PRIOR TO INSTALLING ANY SOFIT PANELS OR RELATED FLASHINGS. DO NOT ALLOW ANY METAL PRODUCTS TO COME INTO DIRECT CONTACT WITH TREATED LUMBER

DAMAGED MATERIAL

Upon determination of responsibility, repair or replace damaged metal panels and trim to the satisfaction of the Architect and Owner.

END OF SECTION