# GUIDE SPECIFICATION

# PAC-850

August 2024

## SECTION 07415- PREFORMED ALUMINUM SOFFIT PANELS, SOLID AND PERFORATED

– GENERAL

DESCRIPTION OF WORK

This section covers the pre-finished, pre-fabricated Factory Manufactured Aluminum Soffit System. All metal trim, accessories, fasteners, insulation and sealants indicated on the drawings are part of this section

Related Work Specified Elsewhere

Roof Deck structural steel, flat roof systems, preformed metal standing seam roofing, perimeter edge systems, roof hatches, firestopping not included in this section

QUALITY ASSURANCE

Petersen Aluminum Corporation products establish a minimum of quality required. Specifier: Choose location:

Petersen Aluminum Corp, Elk Grove Village, IL, 800-323-1960

Petersen Aluminum Corp, Annapolis Junction, MD, 800-344-1400

Petersen Aluminum Corp, Tyler, TX, 800-441-8661

Petersen Aluminum Corp, Acworth, GA, 800-272-4482

Manufacturer and erector shall demonstrate experience of a minimum of five (5) years in this type of project.

SUBSTITUTIONS

The material, products and equipment specified in this section establish a standard for required function, dimension, appearance and quality to be met by any proposed substitution.

SYSTEM DESCRIPTION

Material to comply with:

ASTM B209 Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

SOFFIT SYSTEM PERFORMANCE TESTING

Soffit System shall be designed to meet Standard Building Code wind load requirements.

Soffit System shall be designed to meet applicable Local Building Code and the Soffit System shall have been tested by the Manufacturer per ASTM E-330 and have the applicable Load Tables published from this Air Bag testing for negative loads.

WARRANTIES

Finish warranty: Manufacturer’s standard form in which manufacturer agrees to repair finish or replace soffit panels that show evidence of deterioration of factory-applied finish within specified warranty period.

Exposed Panels Finish – deterioration includes the following:

Color fading more than 5 hunter units when tested according to ASTM D 2244

Chalking in excess of a No. 8 rating when tested according to ASTM D 4214

Cracking, checking, peeling or failure of a paint to adhere to a bare metal.

Warranty Period: 30 Years from the date of substantial completion

SUBMITTALS

Furnish detailed drawings showing profile and gauge of exterior sheets, location and type of fasteners, location, gauges, shape and method of attachment of all trim locations and types of sealants, and any other details as may be required for a weather-tight installation.

Provide finish samples of all colors specified.

LEED Submittals

Product data for Credit MR 4.1 and credit MR 4.2: Indicating the percentages by weight of postconsumer and preconsumer recycled content for products having recycled content

DELIVERY, STORAGE AND HANDLING

Deliver components, sheets, metal soffit panels and other manufactured items so as not to be damaged or deformed. Package metal soffit panels for protection during transportation and handling.

Unload, store and erect metal soffit panels in a manner to prevent bending, warping, twisting and surface damage.

Stack metal soffit panels on platforms or pallets, covered with suitable weathertight and ventilated covering. Store metal soffit panels to ensure dryness. Do not store metal soffit panels in contact with other materials that might cause staining, denting or other surface damage.

Protect strippable protective coating on any metal coated product from exposure to sunlight and high humidity, except to the extent necessary for material installation.

PROJECT CONDITIONS

Weather Limitations: proceed with installation only when existing and forecasted weather conditions permit metal roof panel work to be performed.

Field Measurements: Verify actual dimensions of construction contiguous with metal roof panels by field measurements before fabrication

– PRODUCTS

PANEL DESIGN

Soffit panels shall be 12” wide by ½” deep and manufacturer shall be able to provide all three options of panel surface: Full  Vent, Half Vent or Solid Soffit in the specified color (s).

ACCEPTABLE MANUFACTURERS

This project is specified around the roofing product of Petersen Aluminum Corporation, PAC-850 Soffit.

Other acceptable manufacturers, if they comply with specification:

IMETCO, Tucker, GA, Soffit Panel only

ATAS Aluminum, Allentown, PA, “Wind-Lock” Panel only

MATERIALS AND FINISHES

Materials: ASTM B-209 quality aluminum, 3105-H14 Alloy and Temper material. Aluminum shall be tension leveled (temper passed and stretcher leveled) with camber of a maximum of ¼” in 20 feet, manufactured in the USA, and shall be .032” thick aluminum, US standard grade.

Color shall be PAC-CLAD Kynar 500 in one of 37 standard finishes offered in .032 aluminum:________________________________

Panel Surface shall be:  	Full Vent	Half Vent	Solid

Finishes: Finish shall be Kynar 500 or Hylar 5000 Fluorocarbon coating with a top side film thickness of 0.70 to 0.90 mil over 0.25 to 0.31 mil prime coat to provide a total dry film thickness of 0.95 to 1.25 mil. Finish shall conform to tests for adhesion, flexibility and longevity as specified by Kynar 500 or Hylar 5000 finish supplier.

Field protection must be provided by the Contractor at the job site so material is not exposed to weather and moisture.

If any strippable film coating is applied to any pre-finished panels or materials for protection during shipping, strippable film shall be removed prior to installation.

Forming: use continuous and rolling method. No end laps on panels. No “portable rollforming” machines will be permitted on this project; no installer-owned or installer-rented machines shall be permitted. It is the intent of the Architect to provide Factory-Manufactured soffit systems only for this project.

Trim: Trim shall be fabricated of the same material and finish to match the profiled sheeting and press broken in lengths of 10 – 12 feet. Trim shall be formed only by the manufacturer or their approved dealer. Trim to be erected in overlapped condition. Use lap strips only as indicated on drawings. Miter conditions shall be factory welded material to match the sheeting.

Fasteners: Fasteners shall be 400 series stainless steel, dished washers stainless steel with bonded neoprene.

Zees: Where required by design of primary structural framing system, zees shall be used to span between beams and/or other joists. Thermally responsive base and top clips shall be fastened to the zees on 12” centers.

Insulation: See Section 07210: Building Insulation.

SEALANTS

Provide two-part polysulfide class B non-sag type for vertical and horizontal joints or

One part polysulfide not containing pitch or phenolic extenders or

Exterior grade silicone sealant recommended by roofing manufacturer or

One part non-sag, gun grade exterior type polyurethane recommended by the roofing manufacturer.

FABRICATION

Comply with dimensions, profile limitations, gauges and fabrication details shown and if not shown, provide manufacturer’s standard product fabrication.

Fabricate components of the system in factory, ready for field assembly.

Fabricate components and assemble units to comply with fire performance requirements specified.

Apply specified finishes in conformance with manufacturer’s standard, and according to manufacturer’s instructions.

– EXECUTION

INSPECTION

Examine alignment of structural steel and related supports, primary and secondary roof framing, solid roof sheathing, prior to installation.

For the record, prepare written report, endorsed by installer, listing conditions detrimental to performance of the Work.

Proceed with installation only after unsatisfactory conditions have been corrected.

FASTENERS

Secure units to supports

Place fasteners as indicated in manufacturer’s standards.

INSTALLATION

Panels shall be installed plumb and true in a proper alignment and in relation to the structural framing. The erector must have at least five years successful experience with similar applications.

Install soffit panels, fasteners, trim and related sealants in accordance with approved shop drawings and as may be required for a weather-tight , complete and architecturally pleasing installation.

Remove all strippable coating and provide a dry-wipe down cleaning of the panels as they are erected.

Panels attached to any TREATED LUMBER MUST HAVE AN APPROPRIATE VAPOR BARRIER INSTALLED OVER THE TREATED LUMBER PRIOR TO INSTALLING ANY SOFIT PANELS OR RELATED FLASHINGS. DO NOT ALLOW ANY METAL PRODUCTS TO COME INTO DIRECT CONTACT WITH TREATED LUMBER

DAMAGED MATERIAL

Upon determination of responsibility, repair or replace damaged metal panels and trim to the satisfaction of the Architect and Owner.

END OF SECTION