# GUIDE SPECIFICATION

# Precision Tile Wall

December 2017

## SECTION 07410- METAL WALL TILES

- GENERAL

DESCRIPTION OF WORK

This Section covers the pre-finished, pre-fabricated Factory Manufactured Architectural Concealed attachment Metal Tile Wall system.  All metal trim, accessories, fasteners, insulation, and sealants indicated on the drawings as part of this section.

Drawings and general provisions of the Contract, including general and Supplementary Conditions and Division 01 Specifications, apply to this section

Related Work Specified Elsewhere

Roof Deck structural steel, flat roof systems, preformed metal standing seam roofing, perimeter edge systems, roof hatches, firestopping not included in this section

SUMMARY

Section Includes

Factory formed metal wall tiles

Related work specified elsewhere (Note: select from the below or add appropriate sections)

Metal Roof Deck: Division 5 – Metal Deck Sections

Wood Framing and Decking: Division 6 Roof Carpentry Section

Flashing and Trim: Division 7- Flashing and Sheet Metal

Coping and Gravel Stops: Division 7 Roof Specialties and Accessories

Sealants: Division 7 Joint Sealers Sections

QUALITY ASSURANCE

Petersen Aluminum Corp products establish a minimum of quality required. (Specifier Note: Select from the below)

Petersen Aluminum Corp, Elk Grove Village, IL, 800-323-1960

Petersen Aluminum Corp, Annapolis Junction, MD, 800-344-1400

Petersen Aluminum Corp, Tyler, TX, 800-441-8661

Petersen Aluminum Corp, Acworth, GA, 800-272-4482

Manufacturer and erector shall demonstrate experience of a minimum of five (5) years in this type of project.

Sheet Metal Industry Standard: Comply with Sheet Metal and Air Conditioning Contractors National Association (SMACNA) Architectural Sheet Metal Manual and National Roofing Contractors Association (NRCA) details applicable to wall panels and wall flashings.

Panels shall be factory-produced only. No portable, installer-owned or installer-rented  machines will be permitted

SUBSTITUTIONS

The material, products and equipment specified in this section establish a standard for required function, dimension, appearance and quality to be met by any proposed substitution.

SYSTEM DESCRIPTION (Specifier Note: Select according to required gauge from the below)

ASTM A 653 Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy Coated by the Hot-Dip Process (24 GA G-90 Galvanized Steel)

ASTM B209 Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate (032 Aluminum)

ASTM A240/A240M – Standard specification for Chromium and Chromium Nickel Stainless Steel Plate, Sheet and Strip for Pressure Vessels and for General Applications.

WALL TILE SYSTEM PERFORMANCE TESTING

Air Penetration:  When tested per ASTM  E-283 @ 15 PSF the air penetration shall be .010 or less when tested in accordance here.

Water Penetration: When tested per ASTM E-331 @ 15 PSF for the 15 minute test period, the water penetration shall be none.

Dynamic Water Penetration: When tested per AAMA 501 @ 15 PSF, the water penetration shall be none.

Negative Load Testing per ASTM E-330: The panel shall have been tested per ASTM E-330 to show negative wind uplifts at spans of 1’0” through 4’0” spans, both double and triple spans and the Manufacturer shall provide a Negative Wind Uplift Table for this panel at the above-listed spans, with current 2.0 Safety Factor as per IBC current code and 1.65 Safety Factor as per US Corps of Engineers.

WARRANTIES

Finish warranty: Manufacturer’s standard form in which manufacturer agrees to repair finish or replace wall panels that show evidence of deterioration of factory-applied finish within specified warranty period.

Exposed Panels Finish – For PAC-CLAD Kynar Finish only (stainless steel and anodized finishes excluded)

Color fading more than 5 hunter units when tested according to ASTM D 2244 (varies for Award Blue/Cardinal Red/Weathered Steel, Weathered Copper)

Chalking in excess of a No. 8 rating when tested according to ASTM D 4214 (varies for Award Blue/Cardinal Red)

Cracking, checking, peeling or failure of a paint to adhere to a bare metal.

Warranty Period

30 Years from the date of substantial completion

SUBMITTALS

Furnish detailed drawings showing profile and gauge of exterior sheets, location and type of fasteners, location, gauges, shape and method of attachment of all trim locations and type of sealants, and any other details as may be required for a weather-tight installation.

Provide finish samples of all colors specified.

Shop drawings: Show fabrication and installation layouts of  metal wall panels or metal soffit panels, details of edge conditions, side-seam joints, panel profiles, corners, anchorages, trim, flashings, closures and accessories, and special details. Distinguish between factory and field-assembled work

Coordination Drawings: Roof plans, drawn to scale, on which the following are shown and coordinated with each other, based on input from installer of the items involved:

LEED Submittals

Product data for Credit MR 4.1 and credit MR 4.2: Indicating the percentages by weight of postconsumer and preconsumer recycled content for products having recycled content

DELIVERY, STORAGE AND HANDLING

Ordering: Comply with manufacturer’s ordering instruction and lead time requirements to avoid construction delays.

Deliver components, sheets, metal tiles and other manufactured items so as not to be damaged or deformed. Package metal tiles for protection during transportation and handling.

Unload, store and erect metal tiles in a manner to prevent bending, warping, twisting and surface damage.

Stack metal tiles on platforms or pallets, covered with suitable weathertight and ventilated covering.

Store metal tiles to ensure dryness. Do not store metal wall panels in contact with other materials that might cause staining, denting or other surface damage.

Protect strippable protective coating on any metal coated product from exposure to sunlight and high humidity, except to the extent necessary for material installation.

PROJECT CONDITIONS

Weather Limitations: proceed with installation only when existing and forecasted weather conditions permit metal roof panel work to be performed.

Field Measurements: Verify actual dimensions of construction contiguous with metal roof panels by field measurements before fabrication

- PRODUCTS

PANEL DESIGN

Provide factory-formed metal wall tiles that shall be concealed attachment that are mechanically attached to wall supports and do not have any exposed fasteners on the panel face for attachment to the wall supports. .

Architect to select from the following:

PAC Precision Series Cupped Tile, 14 ½” x 8 5/16” (+/- 1/16”)

PAC Precision Series Flat Tile, 14 ½” x 8 5/16” (+/- 1/16”)

PAC Precision Series Diamond Tile, 7 ¼” x 8 5/16” (+/- 1/16”)

PAC Precision Series TS Tile, 7 ¼” x 16 ¼” (+/- 1/16”)

ACCEPTABLE MANUFACTURERS

This project is detailed around the wall panel product of Petersen Aluminum Corporation, Elk Grove Village, IL : PAC Precision Series Tiles

MATERIAL AND FINISHES

Preformed metal tiles shall be fabricated of  (specifier to choose) 24 GA G-90 Galvanized steel; .032” thick prime aluminum PAC-CLAD Kynar coated; .032” thick prime aluminum PAC 300 Clear Anodized or PAC-500 Bronze Anodized;  26 GA Stainless Steel, 304 or 316, or 26 GA Zalmag. {Architect to select from below options}

Color shall be PAC-CLAD _______________________________. (Specifier – choose finish from available colors shown by gauge on PAC-CLAD color chart)

Finish shall be Kynar 500 or Hylar 5000 Flurocarbon coating with a top side film thickness of 0.70 to 0.90 mil over 0.25 to 0.31 mil prime coat to provide a total dry film thickness of 0.95 to 1.25 mil.  Bottom side shall be coated with a primer with a dry film thickness of 0.25 mil.  Finish shall conform to all tests for adhesion, flexibility and longevity as specified by Kynar 500 or Hylar 5000 finish supplier.

Anodized finish shall be _______________________________. (Specifier – choose finish from available colors shown by gauge on PAC-CLAD color chart)

Clear Anodized Finish to be Exterior Grade #2, .0002 - .0003 thickness.

Stainless Finish to 2B (Mill), BA (Bright Annealed), #4 (Brushed) or #8 (Mirror)

Stainless Colors to be Natural Mill Finish; or Light Interference Colored Stainless _______________________________(Specifier – choose finish from available colors); or TiNi Colored Stainless _______________________________. (Specifier – choose finish from available colors

Zalmag to be Natural or Pre-Patina

Field protection must be provided by the Contractor at the job site so material is not exposed to weather and moisture.

Forming: Precision Tiles to be factory formed only.

Trim: Trim shall be fabricated of the same material and finish to match the profiled sheeting and press broken in lengths of 10 feet. Trim shall be formed only by the manufacturer or their approved dealer. Trim to be erected in overlapped condition. Use lap strips only as indicated on drawings. Miter conditions shall be factory welded material to match the sheeting. Standard Trim pieces to include:

1-Piece outside corner, ¼” exposed faces

3-Piece outside corner, 2” exposed faces

1-piece inside corner, 3” exposed faces

J-Channels – 1” exposed face

Head Flashing – 1 ¾” x 1 ½” with ½” drip

Starting cleat – 2” long with ¼” exposed face

Mounting clips – 3.4” x 3.4”

Accessories/Fasteners: Fasteners shall be of type, material, size, corrosion resistance, holding power and other properties required to fasten miscellaneous framing members to substrates. Accessories and their fasteners shall be capable of resisting the specified design wind uplift forces and shall allow for thermal movement of the wall panel system.

Exposed fasteners shall not restrict free movement of the wall panel system resulting from thermal forces, except at designed points of wall panel fixity.

Fasteners:  Fasteners shall be galvanized steel, dished washers,galvanized steel with bonded neoprene.

Zees:  Where required by design of primary structural framing system shall be used to span between beams and/or joists.

Insulation:  See Section 07 210: Building Insulation.

Sealants

Provide two part polysulfide class “B” non-sag type for vertical and horizontal joints, brand name: NP-1. Geocell 2300, Weathermaster “Titebond” or similar performing caulking.

One part polysulfide not containing pitch or phenolic extenders, or;

Exterior grade silicone sealant recommended by roofing manufacturer, or;

One part non-sag, gun grade, exterior type polyurethane recommended by roofing manufacturer.

2.4	FABRICATION

Comply with dimensions, profile limitations, gauges and fabrication details shown and if not shown and, if not shown, provide manufacturer's standard product fabrication.

Fabricate components of the system in factory, ready for field assembly.

Fabricate components and assemble units to comply with fire and performance requirements specified.

Apply specified finishes in conformance with manufacturer's standards, and according to manufacturer's instructions.

- EXECUTION

3.1	INSPECTION

Examine alignment of structural steel and related supports prior to installation and do not proceed until the defects are corrected by the responsible contractor.

For the record, prepare written report, endorsed by installer, listing conditions detrimental to performance of the Work.

Proceed with installation only after unsatisfactory conditions have been corrected.

3.2	FASTENERS

Secure units to supports.

Place fasteners as indicated in manufacturer's standards.

INSTALLATION

Tiles shall be installed plumb and true in proper alignment and relation to the structural framing.  The erector must have at least five years successful experience with similar applications.

Install metal tiles, fasteners, trim and related sealants in accordance with approved shop drawings and as may be by manufacturer’s installation instructions and details for this wall panel system

Remove all strippable coating and provide a dry wipe-down cleaning of the panels as they are erected.

Panels attached to any TREATED LUMBER MUST HAVE AN APPROPRIATE VAPOR BARRIER INSTALLED OVER THE TREATED LUMBER PRIOR TO INSTALLING ANY SOFIT PANELS OR RELATED FLASHINGS. DO NOT ALLOW ANY METAL PRODUCTS TO COME INTO DIRECT CONTACT WITH TREATED LUMBER

DAMAGED MATERIAL

Upon determination of responsibility, repair or replace damaged metal panels and trim to the satisfaction of the Architect and Owner.

END OF SECTION